Assignement 6

RITA FADLALLAH

Question 1

Considering the same example you solved in the previous assignment (radiative heat transfer between two parallel plates), how many shields with epsilon = 0.1 should you add in order to have the new heat transfer rate to be 1% of the case without shields?

If the values are not equal

𝑄̇𝑓𝑟𝑜𝑚 𝑝𝑟𝑒𝑣𝑖𝑜𝑢𝑠 𝑒𝑥𝑎𝑚𝑝𝑙𝑒 = 3625.4

1 % of Q̇ = 36.254

ε=0.1

36.253=σ(T14-T24)1ε1+1ε2-1-

QN shields= AσT14-T24N+11ε+1ε-1= 1N+1Qno shields

1% of previous case Q= 1100×3625.3Wm2=36.253 Wm2

36.253Wm2=5.67×10-8×(8004-5004)(N+1)(10.1+10.1-1)

36.253Wm2 =19680.57(N+1)(19)

N+1= 28.57

N=27.57~28

If the values are equal

∈1 = ∈2 = ∈3.1 =∈3.2 =0.1

T1 = 800 K

T2=500K

Ǫ A= σT14-T241ε+1ε-1  Ǫ A= 5.67×10-8×(8004-5004)(10.1+10.1-1)  = 1035.81 w/ m2

1035.81 X 1% = 10.35

σT14-T24N+11ε+1ε-1= 1N+1

1N+1Q=  1100  Q

99 SURFACES TO LOWER THE RADIATION 1 %

Question 2

You should create a pdf file with screenshots of all of the steps we went through and explain briefly the reason behind the use of each step.

Step 1: draw a rectangle 40*30

Step 2:  offset 10 towards the center

Step 3: connect the corners to the inner rectangle

Step 4: click on “create spaces from diagrams” and assign 3 to the number of floors.

Step 5: click on “surface matching” and press “match in entire model”

Step 6: go to extensions – openstudio user scripts – alter or add model elements – set window to wall ration

Step 7: press “search surfaces” and assign 90 to 360 in the surface orientation then click on search entire model

Step 8: go to extensions – openstudio user scripts – alter or add model elements – add overhangs by projection factor and press “ok” then save the file

Step 9: select items 101 to 105 the go to “set attributes for selected spaces” and change the parameters as shown below then press “ok”

Step 10: repeat the same procedure from 201 to 205

Step 11: repeat the same procedure from 301 to 305

Step 12: launch openstudio and change the weather data to Piacenza.

Step 13: go to “run simulation” and click run.