Dariush Ahrari

10723063

Task1.

Using the diagrams given in the presentation calculate how much

(%) is the effect of applying different modifications (changing the

gas, adding an extra pane, using a low emissivity coating) on the U

value with respect to a benchmark case of double layer with air

and no coating (keep the gap thickness to be 13 mm)

1. from the graph it is possible to see that by comparing the benchmark with the

first case where the gas (krypton) has been changed, the UFACTOR value

decreases by 7%, thus improving the thermal transmittance of the window.

2. in the second comparison, using a low emissivity coating, the UFACTOR value

decreases by 36%, greatly improving the thermal transmittance compared to

the benchmark.

3. in the last comparison, adding an extra pane, the UFACTOR value, still decreases

by 36%, proving a great improvement in the thermal efficiency of the

window.

Task 2

Consider the house that we analysed in the last two examples,

calculate the heating and cooling load of the other windows

which are fixed 14.4 m2 on the west, fixed 3.6 m2 on the south

and an operable 3.6 m2 on the south (the same window and

frame type). How much does the total value change if I change

the frame of the window from wooden one to aluminium?

PIACENZA

LAT: 44,92 N

LONG: 9,73 E

ELEV :138

TSUMMER: 24°

TWINTER: 20°

HEATING DB 99%: - 4,8

COOLING DB/MCWB 1%: 31,9

Δ𝑇cooling = 31,9 − 24 = 7,9 ℃

Δ𝑇heating= 20 − (−4,8) = 24,8 ℃

EAST SIDE OF THE BUILDING

45° LATITUDE

No internal shading – AIC = 1

DR = 11,9

Wood Frame Section

WINDOW 1

AW1east= 14,4 m2

EAST

FIXED

WOOD FRAME

Heating:

Uw1east= 2,84 W/m2 K

HFw1east= UW1east * ΔTcooling = 2.84 *24.8 = 70.44

Qw1east= HFW1east * AW1east = 70.44 * 14.4 = 1014.2 W

Cooling

Part for Heat transfer

CFW1east = UW1east *(ΔTcooling – 0.46 * DR) = 2,84 (7,9 − 0,46 ⋅ 11,9) = 6,9 W/m2

Part for Irradiation part

ED = 559

Ed= 188

East window of a detached house - FFS = 0.31

SHGC= 0.54

PXIW1east = ED + Ed = 559 + 188 = 747

CFW1east = PXI*SHGC*IAC*FFs= 747*0.54*1*0.31=125.1

CFfenestration1east= Uw1east*(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 6.9 +125.1 =132 W/m2

𝑄̇w1east = CFfenestration1east * AW1east = 132*14.4 = 1900.8 W

WINDOW 2

AW2west= 14,4 m2

WEST

FIXED

WOOD FRAME

Heating:

UW2west= 2,84 W/m2 K

HFW2west= UW2west * ΔTcooling = 2.84 *24.8 = 70.44

QW2west= HFW2west * AW2west = 70.44 * 14.4 = 1014.2 W

Cooling

Part for Heat transfer

CFW2west = UW2west *(ΔTcooling – 0.46 * DR) = 2,84 (7,9 − 0,46 ⋅ 11,9) = 6,9 W/m2

ED = 559

Ed= 188

West window of a detached house - FFS = 0.31

SHGC= 0.54

PXIW2west = ED + Ed = 559 + 188 = 747

CFW2west = PXI*SHGC*IAC*FFs=747*0.54*1*0.56=225.9

CFfenestration2west= Uw2west*(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 6.9+225.9=232.8 W/m2

𝑄̇W2west = CFfenestration2west * AW2west = 232.8 * 14.4 = 3352.32 W

WINDOW 3

AW3south= 3.6 m2

SOUTH

FIXED

WOOD FRAME

Heating:

UW3south= 2,84 W/m2 K

HFW3south= UW3south * ΔTcooling = 2.84 *24.8 = 70.44 W/ m2

QW3south= HFW3south * AW3south = 70.44 * 3.6 = 253.6 W

Cooling

Heat transfer part

CFW3south = UW3south *(ΔTcooling – 0.46 * DR) = 2,84 (7,9 − 0,46 ⋅ 11,9) = 6,9 W/m2

Part for Irradiation part

ED = 348

Ed= 209

South window of a detached house - FFS = 0.31

SHGC= 0.54

PXI W3south = ED + Ed = 348 + 209 = 557

CF W3south = PXI*SHGC*IAC*FFs= 557*0.54*1*0.47=141.4

CFfenestration3south= Uw3south *(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 6.9 +141.4=148.3 W/m2

𝑄̇w3south = CFfenestration3south * AW3south = 148.3*3.6 = 533.88 W

WINDOW 4

AW4south= 3.6 m2

SOUTH

OPERABLE

WOOD FRAME

Heating:

UW4south= 2,87 W/m2 K

HFW4south= UW4south * ΔTcooling = 2.87 *24.8 = 71.17 W/ m2

QW4south= HFW4south * AW4south = 71.17 * 3.6 = 256.2 W

Cooling

Heat transfer part

CFW4south = UW4south *(ΔTcooling – 0.46 * DR) = 2,87 (7,9 − 0,46 ⋅ 11,9) = 6,96 W/m2

Part for Irradiation part

ED = 348

Ed= 209

South window of a detached house - FFS = 0.47

SHGC= 0.46

PXI W4south = ED + Ed = 348 + 209 = 557

CF W4south = PXI*SHGC*IAC*FFs= 557*0.46*1*0.47=120.4

CFfenestration4south= Uw3south *(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 6.9 +120.4=127.3 W/m2

𝑄̇w4south = CFfenestration4south * AW4south = 127.3*3.6 = 458.28 W

______________

𝑄̇Total windows Cooling wood frame = 1900,.8 + 3352.32 + 533.88 + 458.28 = 6245.3 𝑊

𝑄̇Total windows Heating wood frame = 1014.2+1014.2+253.6+256.2=2538.2 W

Aluminium Frame Section

Window 1

AW1east= 14,4 m2

EAST

FIXED

Aluminium FRAME

Heating:

Uw1east= 3.61 W/m2 K

HFw1east= UW1east * ΔTcooling = 3.61 *24.8 = 89.52 W/m2

Qw1east= HFW1east * AW1east = 89.52 * 14.4 = 1289.1 W

Cooling

Part for Heat transfer

CFW1east = UW1east *(ΔTcooling – 0.46 * DR) = 3.61* (7,9 − 0,46 ⋅ 11,9) = 8.7 W/m2

Part for Irradiation part

ED = 559

Ed= 188

East window of a detached house - FFS = 0.31

SHGC= 0.56

PXIW1east = ED + Ed = 559 + 188 = 747

CFW1east = PXI*SHGC*IAC*FFs= 747*0.56*1*0.31=129.6

CFfenestration1east= Uw1east*(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 8.7 +129.6 =138.3 W/m2

𝑄̇w1east = CFfenestration1east * AW1east = 138.3*14.4 = 1991.5 W

WINDOW 2

AW2west= 14,4 m2

WEST

FIXED

Aluminium FRAME

Heating:

UW2west= 3.61 W/m2 K

HFW2west= UW2west * ΔTcooling = 3.61 *24.8 = 70.44

QW2west= HFW2west * AW2west = 89.52 * 14.4 = 1289.1 W

Cooling

Part for Heat transfer

CFW2west = UW2west *(ΔTcooling – 0.46 * DR) = 3.61 (7,9 − 0,46 ⋅ 11,9) = 8.7 W/m2

ED = 559

Ed= 188

West window of a detached house - FFS = 0.56

SHGC= 0.56

PXIW2west = ED + Ed = 559 + 188 = 747

CFW2west = PXI*SHGC*IAC*FFs=747*0.56*1*0.56=234.26

CFfenestration2west= Uw2west*(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 8.7+234.26=242.96 W/m2

𝑄̇W2west = CFfenestration2west * AW2west = 242.96 * 14.4 = 3498.6 W

WINDOW 3

AW3south= 3.6 m2

SOUTH

FIXED

ALUMINIUM FRAME

Heating:

UW3south= 3.61 W/m2 K

HFW3south= UW3south * ΔTcooling = 3.61 *24.8 = 89.52 W/ m2

QW3south= HFW3south * AW3south = 89.52 * 3.6 = 322.2 W

Cooling

Heat transfer part

CFW3south = UW3south *(ΔTcooling – 0.46 * DR) = 3.61 (7,9 − 0,46 ⋅ 11,9) = 8.7 W/m2

Part for Irradiation part

ED = 348

Ed= 209

South window of a detached house - FFS = 0.47

SHGC= 0.56

PXI W3south = ED + Ed = 348 + 209 = 557

CF W3south = PXI*SHGC*IAC*FFs= 557*0.56*1*0.47=146.6

CFfenestration3south= Uw3south *(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 8.7 +146.6=155.3 W/m2

𝑄̇w3south = CFfenestration3south * AW3south = 155.3*3.6 = 559.08 W

WINDOW 4

AW4south= 3.6 m2

SOUTH

OPERABLE

ALUMINIUM FRAME

Heating:

UW4south= 4.62 W/m2 K

HFW4south= UW4south * ΔTcooling = 4.62 *24.8 = 114.57 W/ m2

QW4south= HFW4south * AW4south = 114.57 * 3.6 = 412.4 W

Cooling

Heat transfer part

CFW4south = UW4south *(ΔTcooling – 0.46 * DR) = 4.62 (7,9 − 0,46 ⋅ 11,9) = 11.2 W/m2

Part for Irradiation part

ED = 348

Ed= 209

South window of a detached house - FFS = 0.47

SHGC= 0.55

PXI W4south = ED + Ed = 348 + 209 = 557

CF W4south = PXI*SHGC*IAC*FFs= 557*0.55*1*0.47=143.95

CFfenestration4south= Uw3south *(ΔTcooling – 0.46 * DR) +PXI *SHGC *IAC * FFs = 11.2 +143.98=155.18 W/m2

𝑄̇w4south = CFfenestration4south * AW4south = 155.18*3.6 = 558.65 W

______________

𝑄̇Total windows Cooling Aluminium frame = 1991.5 + 3498.6 + 559.08 + 558.65 = 6607.8 𝑊

𝑄̇Total windows Heating Aluminium frame = 1289.1 + 1289.1 + 322.2 + 412.4 = 3312.8 𝑊

Conclusion

𝑄̇Total windows Cooling Aluminium frame ( 6607 W)  > 𝑄̇Total windows Cooling wood frame ( 6245.3 W)

𝑄̇Total windows Heating Aluminium frame ( 3312.8 W)   > 𝑄̇Total windows Heating wood frame ( 2538.2 W)

Based on the results wood is a better material to use for the frames than aluminium since it has better resistance in cooling and heating aspects